Retaining Wall Design & Verification Report
RT Wall Cantilever
Date: July 18, 2026

This engineering report compiles the retaining wall dimensions, material properties, applied loads, stability safety factors, bearing capacity analysis, FEM settlement profiles, and pile foundation design checks.

# 1. Design Inputs & Parameters

## Wall Geometry

| Parameter | Symbol | Value |
| --- | --- | --- |
| Out-of-plane thickness | t | 1.000 m |
| Wall height | Hw | 3.000 m |
| Toe length | L_toe | 0.800 m |
| Heel length | L_heel | 0.800 m |
| Top wall thickness | b_top | 0.300 m |
| Bottom wall thickness | b_bot | 0.400 m |
| Footing thickness | h_ftg | 0.300 m |

## SNI Typical Geometry Verification Checklist

| Parameter Name | Symbol | Value | SNI Requirement | Status |
| --- | --- | --- | --- | --- |
| Top Wall Thickness | b_top | 0.30 m | >= 0.30 m | PASS |
| Base Stem Thickness | b_bot | 0.40 m | >= 0.33 m | PASS |
| Front Face Batter Slope | slope | 0.0333 | >= 0.0208 | PASS |
| Footing Width | B | 2.00 m | 1.32 ~ 2.31 m | PASS |
| Footing Thickness | h_ftg | 0.30 m | 0.27 ~ 0.33 m | PASS |
| Toe Slab Length | L_toe | 0.80 m | >= 0.67 m | PASS |

## Soil & Water Parameters

| Parameter | Symbol | Value |
| --- | --- | --- |
| Dry unit weight | γ_dry | 18.00 kN/m³ |
| Wet unit weight | γ_wet | 20.00 kN/m³ |
| Friction angle | φ | 45.0° |
| Soil cohesion | c | 10.0 kPa |
| Surcharge load | q | 10.00 kPa |
| Soil height above heel | h_soil | 3.000 m |
| Soil above toe | h_toe | 1.000 m |
| Water unit weight | γ_w | 9.81 kN/m³ |
| Water height behind wall | Hwtr | 0.50 m |
| Water height in front | Hwtr_front | 0.150 m |

# 2. Model & Geometry Diagrams

## Wall Profile & Mesh Grid

## Dimensions Proportions Verification

# 3. Applied Loads & Pressure Distributions

## A. Soil Lateral Pressure (Active & Passive)

Active Earth Coefficient Equation:

Result: Ka = 0.1716

Active Pressure (Top of Stem):

Result: p_act,top = 1.72 kPa

Active Pressure (Bottom of Footing):

Result: p_act,bot = 11.15 kPa

Passive Earth Coefficient Equation:

Result: Kp = 5.8284

Passive Pressure (Bottom of Footing):

Result: p_pass,bot = 68.19 kPa (Mobilized, FS_pass = 2.0)

## B. Soil Vertical Weight & Surcharge

Vertical Backfill Pressure Equation:

Result: p_v,heel = 71.00 kPa

## C. Hydrostatic Water Pressure

Hydrostatic Water Pressure Equation:

Result: Backwater p_w,back = 4.91 kPa, Frontwater p_w,front = 1.47 kPa

## D. Uplift Water Pressure under Base

Base Uplift Pressure Equation:

Result: p_uplift = 4.91 kPa

## E. Pseudo-Static Seismic Forces

## F. Total / Combined Load System

# 4. Stability Analysis & Verification

## Stability Verification Equations

Sliding safety factor equation:

Overturning safety factor equation:

Sliding FS = 72.495 (Required >= 1.5)

Overturning FS = 4.297 (Required >= 1.5)

Global Slope Stability SSRM FS = 1.706

# 5. Bearing Capacity & Settlement

## Bearing Stress & Settlement Equations

Bearing contact stress:

Bearing safety factor:

Consolidation settlement:

Ultimate bearing capacity: 4506.35 kPa

Max bearing stress: 52.98 kPa

Safety factor: 85.05 (Required >= 3.0)

Consolidation Settlement: 94.00 mm (Allowable: 153.33 mm)

# 6. Finite Element Settlement & Base Displacements

Toe Settlement: 23.43 mm

Heel Settlement: 18.14 mm

Differential Settlement: 41.57 mm (Allowable: 50.00 mm)

# 7. Finite Element Stress & Bending Moment Contours

## Stem Node Bending Moment Diagram (kN·m)

## FE Stress Contour σ_xx (kN/m²)

## FE Stress Contour σ_yy (kN/m²)

# 8. Pile Foundation Design & Verification

## Pile Cross Section Geometry & Material Details

| Parameter Name | Symbol | Value |
| --- | --- | --- |
| Pile Material Type | - | Concrete |
| Pile Cross-section Shape | - | Circle |
| Pile Diameter | d_pile | 0.400 m |
| Concrete Cover | d_c | 50.0 mm |
| Main Reinforcement | - | 8 D25 |

## Pile Cross-Section Drawing

## Pile Demands & Safety Factors Checklist

Toe Pile Axial demand: 36.84 kN

Toe Pile Lateral demand: 48.10 kN

Heel Pile Axial demand: 49.75 kN

Heel Pile Lateral demand: 48.10 kN

## Pile Foundation Elevation Schematic

## Pile Winkler Analysis Elastic Profiles

## Moment-Axial Interaction Capacity Curve